Name of the student: Komal Shrikant Jainar

USN: 01FE23BEC298

Div: E

Roll No: 564

EXPERIMENT NO. 1

32  - bit two number addition / subtraction

| Programs | Time in microseconds | Memory space in bytes |
| --- | --- | --- |
| Code 1 AREA ADDITION,CODE,READONLY ENTRY ADD R0,R0,R1; STOP B STOP END | 0.333 | 8 |
| Code 2 AREA ABC,CODE,READONLY ENTRY LDR R0,=0x12345678; LDR R1,=0x87654321; ADD R0,R0,R1; STOP B STOP END | 0.833 | 24 |
| Code 3 TTL ADD32 AREA AA, CODE, READONLY ENTRY MAIN LDR R3, =0x40000000 ;load the starting address LDR R0, [R3] ;get the first data LDR R1, [R3,#0x04] ;get the second data ADDS R2, R0, R1; add the data & store in reg r2 STOP B STOP END | 0.916 | 20 |
| Code 4 TTL ADD32 AREA AA, CODE, READONLY ENTRY MAIN LDR R3, =VAL1 ;load the starting address LDR R0, [R3] ;get the first data LDR R3, =VAL2 ;load the second address LDR R1, [R3] ;get the second data ADDS R2, R0, R1 ;add the data & store in reg r2 STOP B STOP AREA CODE, READONLY VAL1 DCD 0x00000010 VAL2 DCD 0x00000020 END | 1.333 | 40 |

64 – bit two numbers addition / subtraction

| Programs | Time in microseconds | Memory space in bytes |
| --- | --- | --- |
| Code 1 AREA ADDITION,CODE,READONLY ENTRY ADD R4,R1,R0; ADD R5,R3,R2; STOP B STOP END | 0.416 | 12 |
| Code 2 TTL ADD64 AREA AA, CODE, READONLY ENTRY MAIN LDR R0, =0X00000020 ;load 32-bit data LDR R1, =0X00000080 ;load 32-bit data ADDS R2, R0, R1 ;add 32-bit data LDR R3, =0X00000050 ;load 32-bit data LDR R4, =0X00000020 ;load 32-bit data ADCS R5, R3, R4 ;add 32-bit data along with carry STOP B STOP END | 0.748 | 28 |
| Code 3 TTL ADD64 AREA AA, CODE, READONLY ENTRY MAIN LDR R4, =0x40000000 LDR R0, [R4] LDR R1, [R4,#0x04]!; ADDS R5, R1, R0 LDR R2, [R4,#0x04]!; LDR R3, [R4,#0x04]!; ADCS R6, R3, R2 STOP B STOP END | 1.499 | 32 |
| Code 4 TTL ADD64 AREA AA, CODE, READONLY ENTRY MAIN LDR R4, =VAL1 ;load the starting address LDR R0, [R4] ;get the first data LDR R4, =VAL2 ;load the second address LDR R1, [R4] ;get the second data ADDS R5, R1, R0;add the data & store in reg r2 LDR R4, =VAL3 ;load the starting address LDR R2, [R4] ;get the first data LDR R4, =VAL4 ;load the second address LDR R3, [R4] ;get the second data ADCS R6, R3, R2 ;add the data & store in reg r2 STOP B STOP AREA CODE, READONLY VAL1 DCD 0x00000010 VAL2 DCD 0x00000020 VAL3 DCD 0X00000030 VAL4 DCD 0X00000040 END | 2.416 | 76 |

Multiplication of two 32 bit numbers

| Program | Time in microseconds | Memory space in bytes |
| --- | --- | --- |
| Code 1 TTL MUL32 AREA ML, CODE, READONLY ENTRY MULS R2,R1,R0; STOP B STOP END Code 2 TTL MUL32 AREA ML, CODE, READONLY ENTRY MAIN LDR R0, =0X00000005 ; load 32-bit data LDR R1, =0X00000009 load 32-bit data MULS R2, R1, R0 ;multiply the 32-bit data STOP B STOP END | 0.417 0.583 | 8 16 |
| Code 3 TTL MUL32 AREA ML, CODE, READONLY ENTRY LDR R3,=0x40000000; LDR R0,[R3]; LDR R1,[R3,#0x04]!; MULS R2,R1,R0; STOP B STOP END | 1 | 20 |
| Code 4 TTL MUL32 AREA ML, CODE, READONLY ENTRY MAIN LDR R3, =val1 LDR R0,[R3] LDR R3, =val2 LDR R1,[R3] MULS R2, R1, R0 STOP B STOP val1 DCD 0x00004321 val2 DCD 0x00001234 END | 1.500 | 40 |

Division of two 32 bit numbers

| Program | Time in microseconds | Memory space in bytes |
| --- | --- | --- |
| Code 1 TTL DIV32 AREA DIV, CODE, READONLY ENTRY MAIN LDR R0, =0X08 ;load divisor into r0 LDR R1, =0X02 ;load dividend into r1 BACK SUBS R2, R0, R1 ADD R3, R3, #1 CMP R1, R2 BHI STOP MOV R0, R2 b BACK STOP B STOP END | 2.917 | 36 |
| Code 2 TTL DIV32 AREA DIV, CODE, READONLY ENTRY MAIN LDR R4, =dividend LDR R5, =divisor LDR R0, [R4] LDR R1, [R5] MOV R3, #0 BACK CMP R0, R1 BLT STOP SUB R0, R0, R1 ADD R3, R3, #1 B BACK STOP B STOP dividend DCD 0x00000036 divisor DCD 0x00000004 END | 9.250 | 60 |

EXPERIMENT NO. 2

Sum of N 16bit numbers in a given array

| Program | Time in microseconds | Memory space in bytes |
| --- | --- | --- |
| Code 1 TTL SUM16 AREA SUM, CODE, READONLY ENTRY MAIN LDR R0, =VAL1;initialise the sum to zero LDR R1, =0X00 ;initialize the counter LDR R2, =0X05 LOOP LDRH R3, [R0], #0x02 ; pointed by r0 ADD R1, R1, R3 ;add the numbers SUBS R2, R2, #0X01 ;decrement the counter BNE LOOP STOP B STOP AREA VAL, CODE VAL1 DCW 10, 20,30,40,50,60 END | 3.833 | 48 |
| CODE 2 AREA SUM16,CODE,READONLY ENTRY LDR R0,=0X40000000; MOV R3,R4; // No. of values to sum LOOP LDRH R1,[R0],#0X02; ADD R2,R2,R1; SUBS R3,R3,#0X01; BNE LOOP STOP B STOP END | 3.583 | 28 |

Maximum/ Minimum number in given array

| Program | Time in u.sec. | Memory space in bytes |
| --- | --- | --- |
| Code 1 AREA LARGESTNUM,CODE,READONLY ENTRY LDR R0,=ARRAY ; Load the address of the array into R0 MOV R1,#0x05 ; Set number of elements in the array LDR R2,[R0]; Load the first element of the array into R2 (assume it's the largest) ADD R0,R0,#0x04 ; Move to the next element REPEAT SUBS R1,R1,#0x01 ; Decrement the counter BEQ STOP ; If counter is zero, stop LDR R3,[R0],#0x04 ; Load the next element of the array into R3 and post-increment R0 CMP R2,R3 ; Compare R2 with R3 MOVLT R2,R3 ; If R3 is greater than R2, move R3 to R2 BNE REPEAT ; Repeat the loop STOP B STOP ; Infinite loop to stop execution ARRAY DCD 0x11111111,0x22222222,0x33333333,0x55555555,0x44444444 END | 4.583 | 68 |
| Code 2 TTL MAX area largest , code, readonly entry start mov r5,#6 ; intialise counter to 6(i.e. n=7) ldr r1,=value1 ; loads the address of first value ldr r2,[r1],#4 ; word align t0 array element loop ldr r4,[r1],#4 ; word align t0 array element cmp r2,r4; compare numbers bhi loop1 ; if the first number is > then goto loop1 mov r2,r4; if the first number is < then mov content r4 to r2 loop1 subs r5,r5,#1 ; decrement counter cmp r5,#0 ; compare counter to 0 bne loop ; loop back till array ends ldr r4,=result ; loads the address of result str r2,[r4] ; stores the result in r2 stop b stop value1 dcd 0x44444444,0x22222222,0x11111111,0x33333333,0xaaaaaaaa,0x88888888,0x99999999 area data2,data,readwrite result dcd 0x0 end | 7 | 92 |

Factorial of a given number

| Program | Time in u.sec. | Memory space in bytes |
| --- | --- | --- |
| Code 1 AREA FACTORIAL,CODE,READONLY ENTRY LDR R0,=0x00000001; LDR R1,=0x00000005; LOOP MUL R2,R1,R0; MOV R0,R2; SUBS R1,R1,#0x01; BNE LOOP STOP B STOP END | 3.167 | 28 |
| Code 2 area factorial,code entry main mov r0,#5 mov r1,r0 mov r3,r0 loop subs r1,r1,#01 mulne r2,r1,r0 mov r0,r2 bne loop stop b stop end | 3.167 | 32 |
| Code 3 TTL FACTORIAL AREA FACT, CODE, READONLY ENTRY MAIN LDR R0, =dd;load address from lookup table LDR R4, =VAL ;load the input LDR R1, [R4]; SUBS R1,R1,#0X01; MOV R1, R1, LSL #0x02 ;move the shifted data to reg ADD R0, R0,R1;add the data in r0 with shifted data LDR R2, [R0] LDR R3, =0X40000000 STR R2, [R3] STOP B STOP AREA dd, CODE DCD 1 DCD 2 DCD 6 DCD 24 DCD 120 DCD 720 DCD 5040 VAL DCB 5 END | 1.750 | 80 |
| Code 4 AREA SUBFACT,CODE,READONLY ENTRY MOV R0,#0X05; MOV R1,R0; BACK SUBS R1,R1,#0X01; BEQ STOP; BL FACTORIAL; B BACK; STOP B STOP FACTORIAL MUL R2,R1,R0; MOV R0,R2; MOV PC,LR; //also BX LR can be replaced END | 5.417 | 40 |